Additional file 1

Supplementary sensitivity analyses of mechanical heterogeneity and PEEP-inclusive power accounting

Supplement to: Ventilator-Derived Mechanical Power Does Not Uniquely Identify Total Modeled Load or Energy Distribution During Assisted Ventilation: A Two-Compartment Simulation Study.

Sections S1–S5 report the independently varied compliance–resistance analysis. Section S6 reports the separate PEEP-inclusive power-accounting analysis of the 180 primary scenarios. The two analyses retain their respective pressure-reference conventions.

# S1. Design and scope

A supplementary sensitivity analysis independently varied C₂ from 0.02 to 0.10 L/cmH₂O in 0.005-L/cmH₂O increments and R₂ from 4 to 16 cmH₂O·s/L in 1-cmH₂O·s/L increments, while C₁ = 0.04 L/cmH₂O and R₁ = 8 cmH₂O·s/L remained fixed. These illustrative ranges included all primary configurations, lower compliance than the reference, and combinations with opposing compliance and resistance effects, including simultaneously increased C₂ and R₂. Each of the 221 mechanical configurations was evaluated using the same 45 pressure-support, effort-amplitude, and effort-duration combinations as the primary analysis, yielding 9,945 parameter combinations. The original equations, zero initial volume increments, signed pressure–flow integrals, and ventilator-defined inspiratory integration interval were unchanged. Responses of MPtot, EF₁, and EII were examined across the C₂–R₂ grid at fixed loading inputs; effort-amplitude contrasts were also evaluated with mechanical parameters, pressure support, and effort duration held fixed. The full supplementary grid was repeated with dt = 0.0005 s and compared with dt = 0.001 s.

These are illustrative parameter ranges, not estimates of population variability or disease-specific distributions. Resistance ranges from one-half to twice the reference value, and compliance from one-half to 2.5 times the reference value. The primary 180 scenarios remain the main analysis. Their numerical outputs were reproduced exactly at dt = 0.001 s, including 505 ventilator-power-matched pairs and 534 total-power-matched pairs. No additional matched-pair analysis was performed on the enlarged grid.

Each mechanical configuration has 45 parameter combinations but only 39 distinct pressure-input histories: at zero effort amplitude, the three effort-duration labels produce identical passive breaths. The analysis is deterministic and descriptive; these combinations are not independent observations from patients.

# S2. Numerical conventions

The original file assisted_ventilation_model_final.py was imported without modification. Both compartmental volume increments start at zero. The original explicit time integration, half-sine effort input, and square pressure support are retained. All energies use signed flow; negative pressure–flow products within ventilator inspiration are not clipped. Thus these metrics describe net energy transfer over the selected interval, not accumulated positive-only energy or tissue dissipation.

The integration mask is t < 1.0 s. Consequently, the last included sample is 0.999 s at dt = 0.001 s and 0.9995 s at dt = 0.0005 s. The effort waveform is not terminated at 1.0 s in the 1.4-s condition; energy after the ventilator-defined inspiratory interval is excluded. Simulations describe the original initialized single breath, not a repeated-breath steady state or an auto-PEEP model.

# S3. Selected numerical results

Across the supplementary grid, MPtot ranged from 2.53 to 130.21 J/min, EF₁ from 0.289 to 0.767, and EII from 0 to 0.534. The effect of resistance on total modeled power depended on compliance. At pressure support 10 cmH₂O, peak Pmus 6 cmH₂O, and effort duration 1.0 s, increasing R₂ from 4 to 16 cmH₂O·s/L increased MPtot from 17.41 to 18.75 J/min when C₂ was 0.02 L/cmH₂O, but decreased it from 37.50 to 27.40 J/min when C₂ was 0.08 L/cmH₂O.

With opposing mechanical changes (C₂ = 0.08 L/cmH₂O; R₂ = 16 cmH₂O·s/L), EII ranged from 0.008 to 0.090 across loading inputs, compared with a constant value of 0.333 in the primary mixed configuration. At C₂ = 0.02 L/cmH₂O, R₂ = 4 cmH₂O·s/L, pressure support 5 cmH₂O, and effort duration 1.0 s, increasing peak Pmus from 0 to 12 cmH₂O increased EII from 0.315 to 0.534. For C₂ = 0.06 L/cmH₂O and R₂ = 16 cmH₂O·s/L, EF₁ ranged from 0.482 to 0.519 across loading inputs, crossing 0.5. Time-step refinement changed MPtot by at most 0.1565% and EII by at most 0.001203 (Section S5).

## Supplementary Table S1

Fixed-input columns: PS = 10 cmH₂O, peak Pmus = 6 cmH₂O, effort duration = 1.0 s. EII ranges span the original 45 loading-input combinations for each mechanical configuration. All entries use dt = 0.001 s. C₂ units: L/cmH₂O; R₂ units: cmH₂O·s/L; τ₂ units: s; MPtot units: J/min. EF₁ and EII are dimensionless.

| Configuration | C₂ | R₂ | τ₂ | MPtot fixed | EF₁ fixed | EII fixed | EII range |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Symmetric | 0.040 | 8 | 0.32 | 25.00 | 0.500 | 0.000 | 0.000–0.000 |
| Compliance-dominant | 0.080 | 8 | 0.64 | 34.04 | 0.367 | 0.266 | 0.219–0.269 |
| Resistance-dominant | 0.040 | 16 | 0.64 | 23.27 | 0.537 | 0.074 | 0.070–0.123 |
| Mixed | 0.080 | 4 | 0.32 | 37.50 | 0.333 | 0.333 | 0.333–0.333 |
| Severe mixed | 0.100 | 4 | 0.40 | 43.09 | 0.290 | 0.420 | 0.405–0.422 |
| Higher C₂ / higher R₂ | 0.080 | 16 | 1.28 | 27.40 | 0.456 | 0.088 | 0.008–0.090 |
| Higher C₂ / higher R₂ | 0.100 | 16 | 1.60 | 28.47 | 0.439 | 0.122 | 0.036–0.124 |
| Lower C₂ / lower R₂ | 0.020 | 4 | 0.08 | 17.41 | 0.718 | 0.436 | 0.315–0.534 |
| Higher C₂ / higher R₂ | 0.060 | 16 | 0.96 | 25.82 | 0.484 | 0.032 | 0.001–0.038 |

The analysis script exports the complete simulation outputs (9,945 rows per time step), configuration summaries, and selected results as CSV files. Access to the code and data is described in the Availability of data and materials section of the main article. The EF₁ crossing reported above concerns a change in integrated energy preference between loading conditions, not proof of intracycle pendelluft.

# S4. Structural invariance and nonlinear parameter dependence

For each compartment, let xᵢ(t) = Vᵢ(t)/Cᵢ be the elastic recoil pressure. The model equation gives:

dxᵢ/dt = [Pdist(t) − xᵢ(t)] / (RᵢCᵢ).

If the two time constants are equal and x₁(0) = x₂(0) = 0, the two recoil-pressure histories are identical. Hence Q₂(t)/Q₁(t) = R₁/R₂ = C₂/C₁ whenever the ratio is defined, and the same proportionality holds for their integrated energies. For the primary mixed configuration, τ₁ = τ₂ = 0.32 s and C₂/C₁ = 2. Therefore E₂ = 2E₁, EF₁ = 1/3, and EII = 1/3. This is an analytical property of the model equations, verified numerically to floating-point precision, not an additional empirical finding.

A second analytical check illustrates why a model linear in pressure and flow need not respond linearly to its mechanical parameters. For a passive square pressure input of magnitude P, zero initial volume increment, and a continuous integration interval 0 to Tᵢ, the compartmental energy is:

Eᵢ = k P² Cᵢ [1 − exp(−Tᵢ/(RᵢCᵢ))], where k = 0.0980665 J/(cmH₂O·L).

The product RᵢCᵢ enters the exponential, so compliance and resistance have interacting effects even in the passive case. Under time-varying effort, the pressure history additionally interacts with the compartmental response. EII = |2EF₁ − 1| also introduces normalization and an absolute value. Uniform scaling of the entire common pressure waveform preserves energy fractions in the zero-initial-state linear model, but varying muscle-pressure amplitude while pressure support remains fixed is not such a uniform scaling.

# S5. Numerical consistency and precision

| Check | Result |
| --- | --- |
| Supplementary combinations per time step | 9,945 |
| Maximum relative MPtot change on dt refinement | 0.1564368% |
| Median relative MPtot change | 0.0370860% |
| Maximum absolute EII change | 0.001202191 |
| Maximum absolute EF₁ change | 0.000601095 |
| Maximum energy-balance residual | 1.78 × 10⁻¹⁵ J |
| Maximum EF₁ + EF₂ − 1 residual (absolute) | 4.44 × 10⁻¹⁶ |

The time-step figures apply to the supplementary grid and do not replace the smaller errors reported for the original 180 scenarios. They quantify numerical agreement, not physiological validation.

# S6. Sensitivity to inclusion of PEEP in power accounting

## S6.1. Rationale and relation to Gattinoni et al. (2016)

Gattinoni et al. (2016) derived their closed-form mechanical-power equation from a linear equation of motion without an explicit muscle-pressure term and assumed constant flow during inflation. They compared the equation with the work obtained by integrating the inspiratory airway pressure–volume relationship, including PEEP in the pressure reference. The present pressure-support model instead has time-varying, effort-dependent flow and calculates primary energies above PEEP. Therefore, this analysis uses the PEEP-inclusive work integral rather than applying their constant-flow closed-form expression directly to assisted breaths.

For a passive linear single-compartment system at constant flow, the continuous integral reduces to:

MP = k · RR · [Ers · VT2/2 + Raw · VT2/Ti + PEEP · VT].

Here Ers denotes respiratory-system elastance, Raw airway resistance, and VT tidal volume. This is Eq. (6) of Gattinoni et al. after substituting 1/Ti = RR(1 + I:E)/(60 × I:E), apart from rounding of the unit-conversion factor. This reduction establishes the relation to the original equation; it does not validate that closed-form expression under assisted ventilation.

## S6.2. Methods and calculation conventions

All 180 primary scenarios were evaluated without changing the model equations, prescribed inputs, or initial conditions. PEEP remained 5 cmH₂O, RR 18 breaths/min, and Ti 1.0 s. The original signed trapezoidal pressure–flow integration and inspiratory mask (t < Ti) were retained at dt = 0.001 and 0.0005 s. Let I denote this integration window and k = 0.0980665 J/(cmH₂O·L). PEEP-inclusive quantities were calculated as:

MPvent,PEEP = k · RR · ∫I Paw(t) Qtotal(t) dt,

MPtot,PEEP = k · RR · ∫I [Paw(t) + Pmus(t)] Qtotal(t) dt,

Ei,PEEP = k · ∫I [Paw(t) + Pmus(t)] Qi(t) dt.

Muscle-generated power was unchanged. The additional term k · RR · PEEP · ∫I Qtotal(t) dt was added consistently to ventilator and total power. Compartmental energy fractions, EII, ECF, and the total-to-ventilator ratio were recalculated from the PEEP-inclusive energies and stored separately from the primary above-PEEP outputs.

For square pressure support, MPvent,PEEP = (1 + PEEP/PS) MPvent. This is a scenario-level identity, not an independent result. Because PS varied, the rescaling factor was not common to all scenarios. Pairs were therefore rematched separately by MPvent,PEEP and MPtot,PEEP, using the original non-exclusive procedure, scenario ordering, and tolerance |xj − xi|/xi ≤ 0.05 for i < j. Repeated passive entries at the three effort-duration labels were retained. This rematching used the 180 primary scenarios, not the enlarged C₂–R₂ grid.

## S6.3. Results at dt = 0.001 s

PEEP-inclusive ventilator power ranged from 6.16 to 116.10 J/min, and PEEP-inclusive total power from 6.16 to 159.23 J/min. Muscle-generated power ranged from 0 to 43.14 J/min. The PEEP-inclusive total-to-ventilator ratio ranged from 1.00 to 2.50; phenotype-specific maxima were 2.23, 2.30, 2.50, and 2.39 for the compliance-dominant, resistance-dominant, mixed, and severe mixed configurations, respectively.

## Supplementary Table S2. PEEP-inclusive matched-pair comparisons

Absolute differences are reported as median (maximum). All comparisons use the original 5% relative tolerance and dt = 0.001 s. EF₁ and EII are dimensionless and use the PEEP-inclusive compartmental energies.

| Descriptor | Matched by PEEP-inclusive ventilator power | Matched by PEEP-inclusive total power |
| --- | --- | --- |
| Number of matched pairs | 603 | 569 |
| Absolute difference in PEEP-inclusive total power (J/min) | 8.83 (37.13) | 1.05 (5.26) |
| Absolute difference in muscle-generated power (J/min) | 8.46 (36.03) | 7.11 (29.95) |
| Absolute difference in PEEP-inclusive EF₁ | 0.0418 (0.2580) | 0.0735 (0.2574) |
| Absolute difference in PEEP-inclusive EII | 0.0835 (0.3499) | 0.1470 (0.3514) |

For an illustrative comparison within the severe mixed configuration, PS 10 cmH₂O, peak Pmus 12 cmH₂O, and Tmus 1.4 s yielded PEEP-inclusive ventilator power of 69.88 J/min, muscle-generated power of 36.03 J/min, and total power of 105.91 J/min. A passive scenario at PS 15 cmH₂O yielded ventilator and total power of 68.77 J/min. Thus, similar PEEP-inclusive ventilator power coexisted with different total loading within the same mechanical configuration. This is a model comparison, not a clinical example.

## S6.4. Numerical checks

The original above-PEEP pairing counts (505 and 534) were reproduced at the baseline time step. PEEP-inclusive total power equaled ventilator plus muscle power to numerical precision, with a maximum absolute discrepancy below 4 × 10−14 J/min. The compartmental fractions summed to one, and the total-to-ventilator ratio equaled 1/(1 − ECF), within numerical precision.

Refining the time step changed PEEP-inclusive ventilator and total power by at most 0.0742% across the primary scenarios. The maximum absolute change in PEEP-inclusive EII was 0.0001603. At dt = 0.0005 s, PEEP-inclusive matching yielded 604 ventilator-power-matched pairs and 569 total-power-matched pairs. The one-pair change for ventilator matching reflects sensitivity near the 5% boundary; invariance of pair counts under time-step refinement is not claimed (Table S3).

## Supplementary Table S3. PEEP-inclusive numerical verification

Time-step comparisons use dt = 0.001 s as the baseline. Relative changes exclude zero baseline values. These checks describe numerical agreement, not physiological validation.

| Check | Result |
| --- | --- |
| Primary scenarios per time step | 180 |
| Maximum relative change in MPvent,PEEP | 0.0741924% |
| Maximum relative change in MPtot,PEEP | 0.0741924% |
| Maximum absolute change in PEEP-inclusive EF₁ | 0.0000801218 |
| Maximum absolute change in PEEP-inclusive EII | 0.0001602436 |
| Maximum absolute power-balance discrepancy across both time steps | < 4 × 10−14 J/min |
| PEEP-inclusive ventilator-power-matched pairs: dt = 0.001 / 0.0005 s | 603 / 604 |
| PEEP-inclusive total-power-matched pairs: dt = 0.001 / 0.0005 s | 569 / 569 |

## S6.5. Interpretation, limits, and reproducibility

The numerical magnitude of power and its relative source-contribution ratio depends on the pressure-reference convention. Nevertheless, residual differences in total modeled loading and compartmental energy allocation remained after matching on the PEEP-inclusive power measures. Adding the same PEEP term to ventilator and total power necessarily leaves their difference equal to muscle-generated power; that identity is not interpreted as an independent finding.

This analysis tests pressure-reference sensitivity at fixed prescribed mechanics and PEEP. It is not an assessment of physiological responses to PEEP titration, validation against patient data, or a direct application of the original constant-flow closed-form equation to pressure-support ventilation. The original primary calculations, figures, and analyses remain unchanged.

The analysis is implemented in run_R3C1_peep_reference.py, which imports assisted_ventilation_model_final.py without modifying it during execution. Scenario outputs and rematched-pair CSV files for both time steps are provided in supplementary/peep_inclusive/data/ in the study repository. R3C1_summary.json contains descriptive results, algebraic checks, and time-step comparisons. Access to the repository is described in the Availability of data and materials section of the main article.

## Reference for Section S6

Gattinoni L, Tonetti T, Cressoni M, Cadringher P, Herrmann P, Moerer O, et al. Ventilator-related causes of lung injury: the mechanical power. Intensive Care Med. 2016;42:1567–1575. doi:10.1007/s00134-016-4505-2.